Cool Edit 2000

Transform Effects (*.XFM)

Application

Programmer's

Interface

Transform/Generate Effects API  		         Making your own effects modules

Cool Edit can support any number of transform and generate effects.  New effects can be written by following this API.  An effects module is nothing more than a DLL with the following exported functions that Cool Edit calls to do the reading and writing of audio data.

When Cool Edit starts, it checks the program directory for any files ending in .XFM.  If it finds any, it checks to see if the file contains the function QueryXFM, and if so, it calls that function.  If the return value is XFM_VALIDLIBRARY, then it is assumed to be a valid effects module DLL.

The only functions that are required are QueryXfm(), XfmInit(), XfmDestroy(), XfmSetup(), XfmDo(), and some sort of DIALOGMsgProc() for the settings dialog.  A custom structure should be defined to hold all effect specific data.  This structure must also be communicated to Cool Edit so that various variables in the structure can be filled in automatically if you are using the Presets functions.  The COOLINFO structure contains many functions that can be called at any time.  These functions can be accessed easily by using the macros defined in xfmsdll.c.  Functions include reading and writing audio data, progress meter control, cutting and inserting blank data, preset handling, graph handling, FFT functions, and functions for calling other XFM modules.

An internal data structure is used, and defined by the author of the DLL, for storing any relevant information pertaining to the effect.  The structure's format is communicated to Cool Edit through the szStructDef element of XFMQUERY during QueryXfm().  Never use any global variables - any variables you need should either be defined as part of the effect's internal data structure that you define, or allocated locally to the procedure that is using them.  This is important since there may be more than one process using the DLL at any given time.

If you follow the sample code, and use it as a starting point for your own effects module, then you can follow some of these guidelines.  Use the DialogToStruct() function to copy data from your instance data structure to the dialog box.  Use the StructToDialog() function to copy data from the dialog box back to your data structure.  You can leave the CopyToDlgItem() and CopyFromDlgItem() functions essentially unchanged (except for inserting the name of your custom structure).  These two functions copy the handles to your custom data structure, and to the COOLINFO structure to a dialog control so that the instance information can be kept for your instance.  This way, if multiple copies of Cool Edit are using the same effects DLL, then the data will not get scrambled between them.  The CopyToDlgItem() function is called during the dialog box's WM_INITDIALOG, and the CopyFromDlgItem() function is called before any of the data in the custom structure or COOLINFO structure is needed.  Include the file xfmsdll.c to gain access to all the COOLINFO structure functions that are provided to you by Cool Edit.

All XFM functions should be declared as __declspec(dllexport) FAR PASCAL if using Visual C/C++, or the equivalent export definition for any other C compiler.

To support Cool Edit Pro, return 1160 from QueryXfm.

To support Cool Edit 98 and Cool Edit Pro, return 1157 from QueryXfm.  Preview can be implemented in the module, but PreviewStart will fail if being run from Cool Edit 98.

int QueryXfm( lpxq )	Required

XFMQUERY * lpxq	Structure to be filled with all information pertaining to the effect module

This function should fill the XFMQUERY structure with information about the effect module.

Returns:  XFM_VALIDLABRARY if successful, zero otherwise.

The XFMQUERY data structure is defined as:

char 	szName[80]	Text name of the effect that will appear in the menu bar

char 	szCopyright[80]	Any copyright information you care to put in for your DLL

char 	szToolHelp[80]	Text to be shown in toolbar quick-help

WORD 	wSupports	Combination of XFM_ flags for mono and stereo 8 or 16 bit

DWORD 	dwFlags	Combination of the XF_ flags for describing module

DWORD 	dwUserDataLength	Length of transform's internal data structure

char 	szStructDef[96]	Array of chars representing types of data in internal data structure

char 	szPresetDef[96]	Array of 'y' or 'n' representing whether or not to include data item in presets saving

WORD 	wExtra	Number of extra bytes to follow

Constants for the wSupports field:

XFM_MONO8	Effect supports Mono 8-bit data

XFM_STEREO8	Effect supports Stereo 8-bit data

XFM_MONO16	Effect supports Mono 16-bit data

XFM_STEREO16	Effect supports Stereo 16-bit data

XFM_MONO32	Effect supports Mono 32-bit float data scaled to +/-65536.0

XFM_STEREO32	Effect supports Stereo 32-bit float data scaled to +/-65536.0

Constants for the dwFlags field:

XF_TRANSFORM    	This function works with highlighted audio, and belongs in the Transform menu

XF_GENERATE	This function generates new wave data, and belongs in the Generate menu

XF_ANALYZE	This function works with highlighted audio, and belongs in the Options menu.  It does not modify any data, just examines existing data.

XF_SYSTEM	This module should not appear in any menus, as it is a Cool Edit system DLL.

XF_USESPRESETSAPI	Has a presets box (and uses the presets/scripts functions)

XF_USESGRAPHAPI	Has one or more graph controls (and uses the built in graph control functions)

XF_USESFFTAPI	Uses Cool's FFT functions

XF_CONTIGUOUS	Set this if all reads and writes are contiguous (no re-reading over an area allowed)

XF_PREVIEW	(CEPro Only: Uses Preview API for real time interactive previewing of effect.)

XF_USES32BITAPI	Uses new ReadDataEx and WriteDataEx.

XF_MUSTHIGHLIGHT 	If set, function only enabled if a selection is highlighted (most common)

XF_MUSTHAVECLIP    	If set, function is grayed if nothing on clipboard

XF_MODIFIESTOENDOFVIEW	Set this if function modifies data outside user's given selection to end of view

XF_MODIFIESTOENDOFFILE	Set this if function modifies data outside user's given selection to end of file

XF_MODIFIESOUTSIDESEL	Set this if function modifies data outside selection, and fill dwModifyLeft and dwModifyRight upon exiting XfmSetup with the range of data that will be modified.

XF_STRETCHES	Set if result is of a different size and time markers in that area should be proportionally stretched to match.

XF_NOSINGLEEDIT	If set, function is grayed if editing one channel of a stereo waveform (do this if effect changes the size of the highlighted data, eg. stretching)

XF_AUTOOK	During WM_INITDIALOG, this value should be checked, and if it is set, the dialog should send an IDOK message to itself to close the dialog.

XF_AUTOCROSSFADE	If set, and user has global crossfade setting enabled under Settings/System/Data, then beginning and ending of selection is crossfaded with original audio (ideally used to prevent clicks occuring at boundaries of edits). (Uses 32-bit API).

XF_ALWAYSCALLSETUP	Even if the "F3" repeat command without dialog is performed, XfmSetup is still called (and with the XF_AUTOOK flag set)

XF_COLORRED	If set, WM_CTLCOLOREDIT should color text red (used with Limits() to make text red if it is out of range).

Characters used in the structure definition (szStructDef) to tell Cool Edit the data types in the internal effect data structure:

'c'	char

'i'	16-bit int or WORD

'l'	32-bit long or DWORD

'f'	4-byte float

'd'	8-byte double floating point

'g'	HANDLE to a graph (please use the WORD type in your struct definition for graphs)

'h'	HANDLE to globally allocated memory

's'	Array of 256 chars

BOOL XfmInit( ci )	Required

COOLINFO * ci	Cool Edit info structure pointer

This function should do all initialization of the internal data structure that contains all data pertaining to the effect.  The hUserData member of ci is a handle to this data.  Basically, this function should lock the hUserData member, casting it to the internal data type, and fill all the members with default info, and finally unlock the handle.  Data contained in handles to globally allocated memory are not saved in presets.  If a handle is created, and the data pointed to by the handle needs to be saved with a preset, then this must be done by the caller.  One way to do this is to save the filename of a file containing the data in a string in the structure, and then when the preset is called up, load the memory handle with the data from the file (and alternately, save the data to this file when saving a preset).

Returns:  TRUE if all was initialized OK, FALSE if there was an error.

BOOL XfmDestroy( ci )	Required

COOLINFO * ci	Cool Edit info structure pointer

All internally allocated data should be freed (eg globally allocated handles) or destroyed (eg graphs).  Use the hUserData member of ci to access any internal data.

Returns:  TRUE if all was sucessfully destroyed, FALSE otherwise.

BOOL XfmSetup( hWnd, hInst, ci )	Required

HWND hWnd		Handle to a parent window for your setup dialog box

HINSTANCE hInst		Instance handle for this DLL so you can access resources

COOLINFO * ci	Cool Edit info structure pointer

This function should call up a settings dialog.  The dialog box template should be compiled with the DLL, and can be accessed using the hInst parameter.  Below is a sample XfmSetup() function that should suffice for any module.  IDD_TRANSFORM is the dialog box ID, and the dialog box message proc is exported with ordinal 100.  The lParam given to the dialog box proc during WM_INITDIALOG will be a far pointer to the COOLINFO structure.

BOOL FAR PASCAL __export XfmSetup(HWND hWnd, HINSTANCE hInst, COOLINFO *ci)

{	int nRc;

FARPROC lpfnDIALOGMsgProc;

lpfnDIALOGMsgProc = GetProcAddress(hInst,(LPCSTR)MAKELONG(100,0));

nRc = DialogBoxParam(	(HINSTANCE)hInst,(LPCSTR)MAKEINTRESOURCE(IDD_TRANSFORM),

(HWND)hWnd, (DLGPROC)lpfnDIALOGMsgProc,(DWORD)ci);

return nRc;

}

This setup function is called when the user chooses this effect from the menubar, or wishes to call up the settings dialog of the effect for any other reason.  Sorry, there is no support for modeless dialog boxes yet.  The dialog box routine should call EndDialog with TRUE if the user presses OK, or FALSE if they hit Cancel.  Special care must be taken if more than one settings dialog is to be open at one time (if two instances of Cool Edit each have your effect dialog open).  See the example's CopyToDlgItem() and CopyFromDlgItem() functions which copy the relavent instance data to a hidden dialog box text control.

Returns:  TRUE to continue on and call XfmDo to run the function, or FALSE if the user hit Cancel in the dialog box.

BOOL  XfmMultipleWaveSpec (pmws)	Optional – CEPro Only

MWAVESTRUCT * pwms		Cool Edit multiple info structure pointer

When working with multiple waveforms (and XF_MULTIWAVEFORM flag set for this transform) then the following function is called.  Simply fill this structure with the information regarding how many input and output files are needed.

Multiple waveform support is only for CEPro (not CE2000).

The MWAVESTRUCT structure is defined as:

int iSize;

int iMinInputs;

int iMaxInputs;

int iNumOutputs;

iSize will be filled in already, and should be 16, but could be larger in future versions if more parameters are added.

Set iMinInputs and iMaxInputs to define the number of valid input files that are required by your function.  For example, a vocoder requires at least and at most 2 input streams.  A Dolby 5.1 channel encoder may require 6 input streams.  The number of output streams given here will be available for you to use when the XfmDo function is called, and be marked as WAVEINFO_DEST for each file in the pWaveforms list.  If nothing is ever written to the output file, no file is generated for the user when the function is complete.

BOOL XfmDefaultPresets (dwMode, lpWriteIniString, szIniFilename)	Optional

DWORD dwMode		if 0, return version number, else add presets.

FARPROC lpWriteIniString	Use this function to add the presets.

LPSTR szIniFilename	Cool Edit’s COOL.INI file (you can use this, or use your own INI filename)

If dwMode is zero, return a version number (eg 1, 2, 3, etc.) and fill szIniFilename with your transform’s name (the same name you’ll be using when writing the presets themselves).  If that number is newer than the user’s current settings, then new presets will be added by currently calling the function again with dwMode non-zero.  So if dwMode is not zero, add the presets by calling the (*lpWriteIniString)(szXfmName, szPreset, szIniFile) function.

Here’s an example of this function implemented:

#define TRANSFORMNAME "GoofyFunc"

_declspec(dllexport) BOOL FAR PASCAL XfmDefaultPresets(DWORD dwMode, FARPROC lpWriteIniString, LPSTR szIniFilename)

{

if (dwMode==0) // query version and group name

{	lstrcpy(szIniFilename,TRANSFORMNAME);

return 1;

}

(*lpWriteIniString)(TRANSFORMNAME,"Comp 1,3,3, ,500,1,0,1",szIniFilename);

return TRUE;

}

BOOL XfmDo( ci )	Required

COOLINFO * ci	Cool Edit info structure pointer

This function is what does all the actual work.  The COOLINFO structure contains all information necessary for doing the effect, including the portion of the wave that was highlighted, and a handle to this effect's specific internal data structure that should have been filled in during the Setup function.

Returns:  TRUE if all went OK, or FALSE if there was an error.

BOOL DIALOGMsgProc( hWndDlg, Message, wParam, lParam )	Required

HWND hWndDlg			Settings dialog window handle

UINT Message			Message ID to window proc

WPARAM wParam			Windows wParam parameter

LPARAM lParam			Windows lParam parameter

This is the main window procedure for the settings dialog box.  Please look at the sample code for a template of how to create the dialog box procedure.  The main points are described below.

A STATE structure should be used to keep track of information local to this dialog box, such as the handle to the user data, a locked pointer to the data, and a pointer to the COOLINFO structure.  This structure is allocated, locked, and then a pointer to it is set in the window user data area using SetWindowLong.  Upon entering the window proc, GetWindowLong is used to retrieve the pointer to this structure.  The STATE structure looks like:

HANDLE	hThis

HANDLE hTransform

TRANSFORM * pData

COOLINFO * ci

A sample main code fragment looks like:

typedef struct statinfo_t

{	HANDLE hThis;

HANDLE hXfm;

DISTORT * pXfm;

COOLINFO * ci;

} STATE;

__declspec(dllexport) BOOL FAR PASCAL DIALOGMsgProc(	HWND hWndDlg, UINT Message,

WPARAM wParam, LPARAM lParam)

{   STATE * pState= (STATE *)GetWindowLong(hWndDlg,GWL_USERDATA);

if ((pState) && (pState->ci))

{	if (DialogHook(pState->ci,hWndDlg,Message,wParam,lParam))

return TRUE;

if (GraphHook(pState->ci,pState->pXfm->hGraph,hWndDlg,Message,wParam,lParam))

return TRUE;

}

switch(Message)

{	case WM_INITDIALOG:

{	MYXFM * pXfm = NULL;

HANDLE hMyXfm = NULL;

COOLINFO *ci=NULL;

REGISTERGRAPH rgraph;

REGISTERPRESETS rpinfo;

HANDLE hState=GlobalAlloc(GMEM_MOVEABLE|GMEM_ZEROINIT,sizeof(STATE));

STATE * pState = (STATE *)GlobalLock(hState);

pState->hThis=hState;

pState->ci=(COOLINFO *)lParam;

pState->hXfm= ci->hUserData;

pState->pXfm=(MYXFM *)GlobalLock(pState->hXfm);

SetWindowLong(hWndDlg,GWL_USERDATA,(LONG)pState);

pState->ci->hWndDlg=hWndDlg;

...

case WM_DESTROY:

{	HANDLE h;

STATE * pState= (STATE *)GetWindowLong(hWndDlg,GWL_USERDATA);

h=pState->hThis;

GlobalUnlock(h);

GlobalFree(h);

SetWindowLong(hWndDlg,GWL_USERDATA,0);

}

The dialog box procedure should return the following in response to the various buttons that will close the dialog:

IDCANCEL	0	(User decided to cancel all changes made.  All changes are forgotten after the dialog is closed)

IDOK		1	(User decided to accept changes and execute the transform)

IDCLOSE		2	(User decided to close the dialog, but keep the changes made to all settings)

COOLINFO Structure

The COOLINFO structure contains everything you may need for performing a transform effect on existing data, or to create new data.  Following are explanations of the various fields of COOLINFO, though they are not in the same order as declared in the xfms.h file.  The procedure pointers have not been mentioned - see the following section for details on all procedures.

WORD wChannels	Number of channels in audio data (1 or 2)

WORD wBitsPerSample	Bits per sample (8, 16, or 32)

WORD wBlockAlign	Bytes per sample (1, 2, 4, or 8.  Same as wChannels*wBitsPerSample/8).

long lSamprate	Sample rate of given audio data

HANDLE hUserData	Handle to user's internal data structure

HWND hWndDlg	Handle to this dialog box.  This value must be set in WM_INITDIALOG.

DWORD dwLoSample	Starting sample offset to transform

DWORD dwHiSample	Ending sample offset to transform

DWORD dwRightSample	Rightmost visible sample on the current display

DWORD dwLeftSample	Leftmost visible sample on the current display

DWORD dwTimeOffset	If wave has a separate SMPTE offset, this is the number of samples to offset

FARPROC lpTestFunction	Nothing.

XFMQUERY * cq	Structure returned by QueryXfm()

char * szIniFile	INI file to use for storing any data you might want to store.

Char * szHelpFile	HELP file for Cool Edit itself.

HFONT hFont	Handle to a small font that you may use in dialog boxes

BOOL bReplacesHighlightedSelection	If set, when generating audio, newly generated audio will replace highlighted sel.

DWORD dwInsertBlankSamples	If nonzero, this many blank samples will be inserted after OK is hit.

BOOL bHasCoprocessor	Obsolete (doesn’t everybody have one now?)  Set if user has a coprocessor

HINSTANCE hInst	Instance of this DLL

int iScriptFile	Obsolete.  -1 if no script running, else script is running

int iScriptDialogStop	Obsolete.  Nonzero if "Pause at Dialogs" set for Scripts

BOOL * lpProgressCanceled	Pointer to progress canceled flag.  If flag is set, user hit Cancel in progress meter.

WORD wInvalidEntry	Set to the control ID of the last item that was invalid or out of range

WORD wExtraReadFlags	Used automatically in xfmspdll.c to modify flags for reading

WORD wExtraWriteFlags	Same as above, but for writing

DWORD dwFileID	Used automatically in xfmspdll.c for reading and writing, or pass to ReadDataFileEx and WriteDataFileEx

int iNumWaveforms	Number of waveforms if this is a multi-waveform transform

COOLWAVEINFO * pWaveforms	Array of waveform info structures for multi-waveform transforms

The following are all functions defined in xfmspdll.c that wrap the COOLINFO functions, organized by logical groupings.

Reading and Writing Audio Data

To make most effective use of Cool Edit Pro, set the XF_USES32BITAPI flag and do not use the obsolete functions ReadData and WriteData.  Audio can be read in as either 8-bit, 16-bit, or 32-bit float data, and even as mono or stereo regardless of the native format of the audio in Cool Edit.  The wChannels and wBitsPerSample COOLINFO fields will describe one of the requested formats (requested in the XFMQUERY structure’s wSupports flags in QueryXfm).  If you wish to optimize for 16-bit or 32-bit specifically, then set both the XFM_MONO16|XFM_STEREO16 and XFM_MONO32|XFM_STEREO32 flags in QueryXfm, and then poll wBitsPerSample for the ideal data format, and be sure to ask for data in this format.

ReadDataEx and WriteDataEx take a formatting flag as the last parameter, which is a combination of the RWEX_ flags.  The RWEX_CLIP and RWEX_DITHER flags are set automatically by Cool Edit depending on the user’s main data preferences, and other criteria, but if used, data will forcibly be clipped or dithered.  The 32-bit format is a 4-byte float, scaled to the range of +/-32768.0 (basically the (float) representation of the corresponding short int 16-bit sample).  Other floating point formats may be added in the future, and RWEX_ flags associated with them.

A transform of the data is performed in the XfmDo function by reading data into a buffer, starting at the ci->dwLoSample offset, modifying the data in any way necessary, then writing the data back.  The Reading and Writing method was chosen over Getting and Releasing a data buffer because of the flexibility of data types – a transform only needs to be written to handle 32-bit floating point mono and stereo data and it will work flawlessly with 8, 16, or 32-bit data in Cool Edit.  Plus, if the Cool Edit file is 16-bit, the 32-bit result can be dithered cleanly back to 16-bit if the user wishes.  Of course, if the XFM only handles 16-bit data (as in the Distort example), it will still work with 32-bit audio data in Cool Edit.

The following flags can be OR’d together and used in the wFlags parameter to describe the data type you wish to work with:

RWEX_MONO		RWEX_8BIT

RWEX_STEREO		RWEX_16BIT

RWEX_32BIT

The buffer used to hold the data should be large enough to hold the number of samples necessary.  To calculate, you can use the following macro, or something similar:

dwBytes=dwLength*((wFlags&RWEX_STEREO)?2:1)*((wFlags&RWEX_16BIT)?2:((wFlags&RWEX_32BIT)?4:1))

Two extra flags may be used if the audio data is known to be stereo, and your buffer contains or will contain mono data.  For reading, they are RWEX_FROMLEFT and RWEX_FROMRIGHT.  For writing they are RWEX_TOLEFT and RWEX_TORIGHT.

The functions XfmMultipleWaveSpec, ReadDataFileEx, and WriteDataFileEx can be used to implement multiple waveform capable transforms.

DWORD ReadDataEx( ci, pData, lOffset, dwSize, wFlags )

COOLINFO * ci		Cool Edit info structure pointer

char * pData		Buffer to read into

long lOffset		Offset into caller’s wave data

DWORD lSize		Number of samples to read

WORD wFlags		Data type to read as

Read dwSize samples into the buffer pData from the user's waveform data starting at lOffset samples into the Cool Edit waveform.  Cool Edit tries to read the entire amount always, and will pad with silence where necessary.  Remember that the this functions works in units of samples, not bytes, and that the formula for converting samples to bytes is Bytes = Samples * wBitsPerSample * wChannels / 8.

Returns:  Number of bytes read.

Note 1:  The user’s selection is from ci->dwLoSample and ci->dwHiSmaple.  It is OK to read beyond this (even if the data read would be before the beginning of the file or after the end of the file) to gain context for the operation.

Note 2:  Preview works best if you only read samples from ci->dwLoSample to ci->dwHiSample.

Note 3:  Preview will only work properly if Reading and Writing are contiguous and non-overlapping.

DWORD WriteDataEx( ci, pData, lOffset, dwSize, wFlags )

COOLINFO * ci		Cool Edit info structure pointer

char * pData		Buffer to write out

long lOffset		Offset into caller’s wave data

long lSize		Number of samples to write

WORD wFlags		Data type to read as

Write dwSize samples from the buffer pData to caller’s waveform data starting at lOffset samples into the data. Data at offsets below zero are ignored, and data at offsets greater than the length of the file extend the length of the Cool Edit file.  Remember that the this functions works in units of samples, not bytes!

Returns:  Number of samples written.

DWORD ReadDataFileEx( ci, dwFileID, pData, lOffset, dwSize, wFlags )	CEPro Only

DWORD WriteDataFileEx( ci, dwFileID, pData, lOffset, dwSize, wFlags )

COOLINFO * ci		Cool Edit info structure pointer

DWORD dwFileID		ID of file to access

char * pData		Buffer to write out

long lOffset		Offset into caller’s wave data

long lSize		Number of samples to write

WORD wFlags		Data type to read as

When working with multiple input and/or output files, the specific file id must be given to read from or write to.  The file id is given in ci->pWaveforms[n].dwFileID where n ranges from 0 to ci-> iNumWaveforms-1.  The functions work exactly like ReadDataEx and WriteDataEx in other respects.

Multiple input and output streams is only supported for Cool Edit Pro (not Cool Edit 2000).

Returns:  Number of samples read or written.

The pWaveforms member of the COOLINFO structure contains information about each of the files that are possible to use.

The COOLWAVEINFO structure is defined as:

DWORD	dwFlags;

WORD	wChannels;

WORD	wBitsPerSample;

long	lSampleRate;

DWORD	dwLoSample;

DWORD	dwHiSample;

DWORD	dwFileID;

char	szName[128];

Generally the wChannels, wBitsPerSample, and lSampleRate values will be the same for all files since these multiple file transforms are only called from within the multitrack editor.  The dwFlags parameter contains either WAVEINFO_SOURCE or WAVEINFO_DEST designating the purpose of the file as a source or destination file.  The dwLoSample and dwHiSample values are used internally (in xmfspdll.c) to determine the highlighted portion of the waveform which should be affected.  The lOffset used when calling these functions should be the same you would use for normal single-waveform functions (ie use ci->dwLoSample to ci->dwHiSample).  Nothing stops you from using values outside this range, but this is the range the user has selected.  Default names are placed in the szName string (eg "New Track 7”) but they can be changed to any name you like.

BOOL DelayWriteInitEx( ci, lWriteBehindSize, lOffset, lFullLength, wFlags )

COOLINFO * ci		Cool Edit info structure pointer

long lWriteBehindSize		minimum amount of delay before writing

long lOffset		Offset into caller’s wave data to start writing

long lFullLength		Total number of samples that will be written

WORD wFlags		Data type to read as

Sometimes when writing data, the data written back will overlap data that is yet to be read, or you may wish to re-read data and want to read the previous original data that was there, not the data that was just written back.  This function will delay the actual writing of the data to the file by the number of samples given in lWriteBehindSize.  The restriction to writing data this way is that writing must be sequential.  Data will start being written at lOffset, and continue sequentially untill all lFullLength samples are written.  Also, if working with multiple output streams, only one stream can be delay-written to at a time.

Returns:  TRUE if successful, FALSE if lWriteBehindSize is too large.

DWORD DelayWriteEx( ci, pData, lNumSamples)

COOLINFO * ci		Cool Edit info structure pointer

void * pData		Data to be written

long lNumSamples		Number of samples to write

Writes lNumSamples samples of data to the output stream.  The offset being written to has already been determined by the call to DelayWriteInitEx and the previous calls to DelayWriteEx.

Returns:  The number of samples written.

Void DelayWriteDestroyEx( ci )

COOLINFO * ci		Cool Edit info structure pointer

Writes out all pending data in the delay line to the output file, and frees all internal structures and memory allocated for the delay write process.

DWORD CutSamplesEx( ci, dwOffset, dwSamples)

COOLINFO * ci		Cool Edit info structure pointer

DWORD dwOffset		Offset into caller’s wave data

DWORD dwSamples		Number of samples to remove

Removes samples from waveform file.  This will shorten the caller’s waveform file by the number of samples given.

Returns:  Number of samples removed.

DWORD InsertSamplesEx( ci, dwOffset, dwSamples, bClear)

COOLINFO * ci		Cool Edit info structure pointer

DWORD dwOffset		Offset into caller’s wave data

DWORD dwSamples		Number of samples to remove

BOOL bClear		If set, clears the newly inserted data to silence

Insert samples from waveform file.  This will lengthen the caller’s waveform file by the number of samples given.

Returns:  Number of samples inserted.

Progress Meter

If a function is going to take any length of time, a progress meter should be used so the user can see how long the function is going to take, and to provide a method for the user to cancel the operation.  These functions should be used in the XfmDo function, and anywhere else length operations are going to take place (such as in analyzing data, etc.).

void ProgressCreate( ci, cszText, hwndParent )

COOLINFO *ci			Cool Edit info structure pointer

LPCSTR cszText			Progress message to display above meter

HWND hwndParent			Parent window of meter display

Create a progress meter to show the user how much of the function has been processed.  This meter automatically estimates the total time the function is anticipated to take.  cszText contains text that will be displayed at the top of the meter window, for example, "Flanging Selection...".  Set hwndParent to NULL to have the Cool Edit main window be the parent (most common).

BOOL ProgressMeter( ci, dwCurrent, dwTotal )

COOLINFO *ci			Cool Edit info structure pointer

DWORD dwCurrent		Measure of current amount done

DWORD dwTotal			Measure of total amount to do

The progress bar is updated with the percentage 100 * dwCurrent / dwTotal.  Usually dwTotal is the total number of bytes that your function is going to process, and dwCurrent is the number of bytes processed so far when the function is called.  The progress meter must have been created with ProgressCreate() before calling this function.

At the same time ProgressMeter is called (or just after), you should also check the *ci->lpProgressCancelled flag, and if set, break out of the main loop and exit XfmDo.

ProgressMeter(ci,dwAmountProcessed,dwTotalAmount);

if (*(ci->lpProgressCanceled))

break;

Returns:  FALSE if user hit Cancel, TRUE if all is going OK.

BOOL SetProgressText( ci, szText )

COOLINFO *ci			Cool Edit info structure pointer

LPCSTR szText			Text to place inside progress meter window

Specific information about the transform being performed can be placed in the progress meter dialog itself.  For example, the Click/Pop/Crackle Eliminator function reports the number of clicks found and repaired in this space.

Returns:  TRUE if all went well.

void ProgressDestroy( ci )

COOLINFO *ci			Cool Edit info structure pointer

Close the progress meter box.  Always remember to close the progress meter if you created one when processing is finished.

User Presets

User presets allow for the saving and recalling of often used settings.  A set of pre-defined presets can also be generated by using the WritePrivateProfileString function in response to the PRESETN_NOPRESETS notification message.  The bare minimum that is required to add preset capabilities to your XFM module is to call RegisterPresets with a bounding rectangle for the presets controls during WM_INITDIALOG and respond to the PRESETN_ADDING and PRESETN_CHOSEN notification messages.  The rest is automatic.

BOOL RegisterPresets( ci, prp)

COOLINFO *ci			Cool Edit info structure pointer

REGISTERPRESETS * prp		Pointer to REGISTERPRESETS struct with initialization info

Fill the REGISTERPRESETS struct with pertinent information – all members must be filled out.  This function should be called from within WM_INITDIALOG.  The structure contains:

DWORD dwSize		Just set to sizeof(REGISTERPRESETS)

char szGroupName[64]	The group entry name in COOL.INI to place the presets under

HWND hWndDlg		Set this to the main settings dialog

UINT uiListBox		ID of a List Box to hold names of presets, or zero for automatic

UINT uiAddButton		ID of “Add” button, or zero for automatic

UINT uiDelButton		ID of “Del” button, or zero for automatic

UINT uiNotify		WM_COMMAND id to receive notification messages

UINT uiRect		Zero, or ID or bounding rectangle for automatic

char szPresetHeading[32]	Empty string, or replacement name for “Presets” heading

Generally, you’ll want to go the automatic route, by filling in zero for the uiListBox, uiAddButton, and uiDelButton fields, and giving the ID of the bounding rectangle to uiRect in which the controls will go.  Doing this will give your dialog a more standard look that will match with the rest of Cool Edit.  If you wish to customize the locations of the list box and Add and Delete buttons, then you must fill in the three Ids accordingly.

Returns:  TRUE if presets were started OK.

Notes:  You will need to respond to a few of the notification messages for Presets to be fully functional.  The WM_COMMAND Message will have LOWORD(wParam)==uiNotify, and HIWORD(wParam)==Notification Message.  To do this, set up the notification code as in the example WM_COMMAND entry below:

case ID_NOTIFY:

switch (HIWORD(wParam))

{	case PRESETN_NOPRESETS:

WritePrivateProfileString(TRANSFORMNAME,"Item1","My Effect,3,2,0,0",ci->szIniFile);

WritePrivateProfileString(TRANSFORMNAME,"Item2","Cool Thing,3,7,1,1",ci->szIniFile);

break;

case PRESETN_CHOSEN:

StructToDialog(ci, lpAmp, hWndDlg);

if (bPreviewing)

PreviewUpdate(ci, PREVIEW_NORESTART);

break;

case PRESETN_ADDING:

DialogToStruct(ci, hWndDlg, lpAmp);

break;

}

break;

When a preset is chosen, the settings from the lpAmp structure (which is a locked down pointer to the user defined XFM data structure) are copied to the dialog box to update all the controls in PRESETN_CHOSEN.  When a preset is about to be added, all the data from the controls is copied to the lpAmp structure first in PRESETN_ADDING.

PRESETN_ADDING	Sent before a preset is about to be added to the presets list

PRESETN_ADDED		Sent after a preset has been added to the presets list (lParam==index of item added)

PRESETN_DELETING	Sent before a preset is about to be deleted (lParam==index of item to delete)

PRESETN_DELETED	Sent after a preset has been deleted

PRESETN_CHOOSING	Sent before a preset is about to be chosen (lParam==index of item being chosen)

PRESETN_CHOSEN	Sent after a preset has been chosen (lParam==index if item chosen)

PRESETN_NOPRESETS	Sent during registration if there are no presets defined yet (define your defaults now).  Alternately, you can implement the XfmDefaultPresets function which also allows you to update a user’s preset list with new presets, even if some already exist.

PRESETN_INITIALIZING	Sent before presets are to be initialized

PRESETN_INITIALIZED	Sent after initializing is complete

PRESETREPLY_CANCEL	Return from an ADDING or DELETING to cancel operation

If you wish to manipulate the controls that are filled in automatically, the resource IDs for the new controls are 11500 (list box), 11501 (Del button), 11502 (Add button), and 11503 (“Presets” text).

BOOL XfmDefaultPresets( dwMode, lpWriteIniString, szIniFile)

DWORD dwMode			Zero if being queried, else non-zero

FARPROC lpWriteIniString		Function to use to write out your preset settings if saving to cool.ini

LPSTR szIniFilename		Path to COOL.INI file to write settings to

If called with wParam==0, then szIniFilename should be filled with the transform name (this should be the section in the INI that will contain the presets).  Then return a version number.  Normally just return 1.  If you add new presets, and want them to automatically show up next time the xfm is run, then increase the version number when new presets are added.

If called with wParam!=0, then write out all the default presets.  To save them into cool.ini, use the following format:

(*lpWriteIniString)(“My Transform”, “Effect 1,3,1,2,3,4,5”, szIniFilename);

BOOL DialogHook( ci, hWndDlg, wParam, lParam)

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

WPARAM wParam			wParam value of dialog proc

LPARAM lparam			lParam value of dialog proc

To make the presets work, Cool Edit needs to intervene and monitor the various messages coming in to your dialog box procedure.  This is done via the DialogHook, which you simply place near the top of your dialog box code as shown:

STATE * pState= (STATE *)GetWindowLong(hWndDlg,GWL_USERDATA);

if ((pState) && (pState->ci))

{	if (DialogHook(pState->ci,hWndDlg,Message,wParam,lParam))

return TRUE;

}

When Cool Edit completely handles the message, it will return TRUE from DialogHook, otherwise the message may need further processing from your dialog box procedure.  Since the COOLINFO structure is needed to make the call to DialogHook, it must be filled in and valid before making the call, which is the reason for the validity check in the example above.  The GWL_USERDATA is zero by default when a dialog box is created, so DialogHook will not be called then.  GWL_USERDATA is set to the STATE structure in WM_INITDIALOG, so the DialogHook function never gets called before WM_INITDIALOG.  The DialogHook function replaces all earlier HandleID_XXX functions from earlier versions of Cool Edit.

Graph and Display objects

Common graphical entry and display objects are available for you to use, and using them brings more coherence between your XFM and the rest of Cool Edit.  The graph object is the device where you click on points and drag them around to create a line that follows y=f(x) (see Envelope or Distortion for an example) where there is one and only one y value for any x value (that is, the graph can not loop back on itself).  The Display object just displays a graph only, and has no user controls over it, but it can display multiple colored lines (see Scientific Filters for an example).  Sometimes a single numerical value is not enough for some functions – in most cases a graph control will do the trick.

BOOL RegisterGraph ( ci, pRegisterGraph )

COOLINFO *ci			Cool Edit info structure pointer

REGISTERGRAPH pRegisterGraph	Pointer to REGISTERGRAPH struct to use for initialization

As many graph controls as needed may be used in a single settings dialog.  Call RegisterGraph for each one.  Each field must be filled out completely, with unused fields left as zero.  The REGISTERGRAPH structure has the following members:

DWORD dwSize		Just set to sizeof(REGISTERGRAPH)

HWND hWndDlg		Set to the settings dialog handle

UINT uiNotify		WM_COMMAND ID that will be used for notification messages

UINT uiRect		ID of Bounding rectangle

UINT uiDisplayText		ID of a static text control for displaying cursor location information

WORD hGraph		Handle to the graph itself (created by calling GraphCreate inside XfmInit usually)

Notification messages will be sent to the WM_COMMAND ID given in uiNotify (with LOWORD(wParam)==uiNotify, and HIWORD(wParam)==Notification Message).  In all cases, the lParam is the index of the point that was clicked on, which will be between zero and GraphCount() -1

GRAPHN_LBUTTONDOWN		User clicked down on a point

GRAPHN_RBUTTONDOWN		User right-clicked down on a point

GRAPHN_BUTTONUP		User lifted up on button over a point

GRAPHN_MOUSEMOVE		User is moving a point with the mouse

GRAPHN_DBLCLK			User double-clicked on a point

Generally, you’ll want to update the real-time preview during the GRAPHN_LBUTTONUP and perhaps even the GRAPHN_MOUSEMOVE notifications to keep the audio in sync with what is being displayed.

Return value: TRUE if registered OK.

BOOL GraphHook( ci, hGraph, hWndDlg, wParam, lParam)

COOLINFO *ci			Cool Edit info structure pointer

WORD hGraph			Handle to graph control

HWND hWndDlg			Handle of settings dialog

WPARAM wParam			wParam value of dialog proc

LPARAM lparam			lParam value of dialog proc

To make the graphs work properly, Cool Edit needs to intervene and monitor the various messages coming in to your dialog box procedure.  This is done via the GraphHook, which you simply place near the top of your dialog box for each graph control in your XFM as examplified below:

STATE * pState= (STATE *)GetWindowLong(hWndDlg,GWL_USERDATA);

if ((pState) && (pState->ci))

{	if (GraphHook(pState->ci,pState->lpAmp->hGraph1,hWndDlg,Message,wParam,lParam))

return TRUE;

if (GraphHook(pState->ci,pState->lpAmp->hGraph2,hWndDlg,Message,wParam,lParam))

return TRUE;

}

When Cool Edit completely handles the message, it will return TRUE from GraphHook, otherwise the message may need further processing from your dialog box procedure.  Since the COOLINFO structure is needed to make the call to GraphHook (just as in DialogHook above), it must be filled in and valid before making the call, which is the reason for the validity check in the example above.  The GWL_USERDATA is zero by default when a dialog box is created, so GraphHook will not be called then.  GWL_USERDATA is set to the STATE structure in WM_INITDIALOG, so the GraphHook function never gets called before WM_INITDIALOG. The GraphHook function replaces all earlier GraphHandle_XXX functions from earlier versions of Cool Edit.

HANDLE GraphCreate ( ci, iLeft, iRight, iMin, iMax, iLeftVal, iRightVal )

COOLINFO *ci			Cool Edit info structure pointer

int iLeft				Leftmost graph coordinate (along x axis)

int iRight				Rightmost graph coordinate

int iMin				Minimum graph value (along y axis)

int iMax				Maximum graph value

int iLeftVal			Value of leftmost point (must be between iMin and iMax)

int iRightVal			Value of rightmost point (must be between iMin and iMax)

This function returns an handle to a graph that is passed to subsequent graph functions.  Graphs are usually created in the XfmInit() function, and destroyed in the XfmDestroy() function.  A graph control allows the user to specify any number of points connected by a single line that has only one y value for every x value.

Returns:  Handle to graph if successful, or NULL if no graph was created.

int GraphCount ( ci, hGraph )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

The user can create new points in a graph at any time.  This function returns the total number of points in the graph.

Returns:  The number of points in the graph.

POINT GraphGetPoint ( ci, hGraph, iIndex )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

int iIndex				Point to get (0 to GraphCount() - 1)

Returns a point structure (whose members are .x and .y) that contains the x and y coordinates of the point at the given index.  Index values range from 0 for the left hand point on up to the number of points in the graph minus one for the rightmost point.

Returns:  x and y value of point in graph.

void GraphSetPoint ( ci, hGraph, int iIndex, ptPoint, bEndPoint )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

int iIndex				Index of point to set

POINT ptPoint			Coordinates of point to add or set

BOOL bEndPoint			TRUE if this point is the rightmost point (as in adding new points to the end)

Call this function to add new points or change the position of points in the graph.  Not used very often.  You can build a set of points for a graph by using this function, but usually the user is the one who creates the points by clicking and dragging.  If the point you are adding is a new rightmost point, indicate by setting bEndPoint to TRUE.

double GraphGetValueAt ( ci, hGraph, fXValue )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

double fXValue			X coordinate on graph

Returns the y value at any given point along the x axis of the graph.  Values between points on the graph are linearly interpolated, or spline-wise interpolated, so no matter what x value is given (provided it is between zero and one) a valid value is returned.  The function works with double precision floating point values since the y value is interpolated, and may not be an integral value.  Only values between zero and one should be passed in.  Zero corresponds to the leftmost side of the graph, while 1.0 corresponds to the right hand side of the graph.  Any X axis coordinate must be scaled down to this range then before calling GraphGetValueAt.  The return value will be in graph coordinates, that is, between the limits (iMin and iMax) given in the GraphCreate call.

To convert the X coordinate from graph values (between iLeft and iRight) and the normalized x value, use the following conversion:

x = (double)(iValue-iLeft)/(double)(iRight-iLeft)

Returns:  The y value at the given x location in graph coordinates.

void GraphDraw ( ci, hGraph, hDC )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

HDC hDC				Device Context of dialog box for drawing graph into

This function should be called in the WM_PAINT routine of the settings window.  The hDC should be the same hDC that was returned from BeginPaint().  The code snippet blow illustrates getting the pointer to the user defined structure (lpMyVars), and then getting the dialogs device context, drawing the graph, end ending the paint routine.  The graph is a HANDLE type in the users structure, and has the code 'g' associated with it in the definition variable szStructDef.

case WM_PAINT:

{	PAINTSTRUCT ps;

HDC hDC=BeginPaint(hWndDlg,&ps);

GraphDraw(pState->ci,pState->lpAmp->hGraph,hDC);

EndPaint(hWndDlg,&ps);

return FALSE;

}

GraphDraw may also need to be called at other times, for example during the StructToDialog call, or any time you notice the graph is not being updated after making a particular change.

void GraphClear ( ci, hGraph )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

Clear all the points in the graph except for the leftmost and rightmost endpoints.

void GraphInvert ( ci, hGraph )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

Invert the graph if the graph is indeed invertable.  The graph is invertable if there is exactly one x point for every y point.  When the graph is inverted, every x value is swapped with every y value.  The property that there is only one y value for each x value must still hold.  Sorry, but no indication is returned if the function inverted the graph or not.

void GraphCopy ( ci, hGraphDest, hGraphSource )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraphDest		Handle to graph to copy into

HANDLE hGraphSource		Handle of graph to copy from

Copy the contents of the source graph to the destination graph.  Both graphs must have already been created using GraphCreate().

void GraphSetDblClkScales ( ci, hGraph, fScx, fOfx, fMagx,

fScy, fOfy, fMagy, wStyle )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

double fScx			Scaling factor for X coordinate

double fOfx			Offset for X coordinate

double fMagx			Precision of X coordinate

double fScy			Scaling factor for Y coordinate

double fOfy			Offset for Y coordinate

double fMagy			Precision of Y coordinate

WORD wStyle			Style (see below)

Set the scaling factors that the actual points will be modified by before being displayed in the text box.  The text box whose ID was given in the GraphSetDialog() call will display the value of the point when a point is selected.  The value displayed is not the same as the integer value of the point, but a value base on the following formula:  DisplayValue = PointValue * fScx + fOfx.  The value of the point is multiplied by the scaling factor and added to the offset to get the final value displayed to the user.  The magnitude/precision value tells how many decimal places to round the value off to.  For example, fMagx of 10 will round values to one decimal place (eg 3.1), while fMagx of 100 will round values to two decimal places (eg 3.14).  Values that are not powers of 10 will round according, for example fMagx of 4 will give values such as "4", "4.25", "4.5", and "4.75".

The actual data displayed to the user will be this value along with the units given in the GraphSetDblClkNames() function.  The wStyle parameter should always be zero.  If it is set to 1, then the final value is converted to decibels (eg 0.5 becomes -3, and 2.0 becomes +3).

The style can be any combination of (or none of) the following:

GRAPH_LOGXAXIS	The graph is divided up logarithmically from left to right (iLeft during GraphCreate must not be zero)

GRAPH_SPLINES		If set, smooth spline curves are drawn between the points instead of straight linear lines.

GRAPH_HASRULERS	If set, horizontal and vertical rulers with grid are displayed as well.

void GraphSetDblClkNames ( ci, hGraph, cszXText, cszXUnits,

cszYText, cszYUnits )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

LPCSTR cszXText			Name for values along x axis (going left and right)

LPCSTR cszXUnits			Units for values along x axis

LPCSTR cszYText			Name for values along y axis (going up and down)

LPCSTR cszYUnits			Unints for values along y axis

Set the names for the values along the x and y axes, for example, "Input Signal Level" or "Frequency", and the names for the units of these, for example, "dB" or "Hz".  The units given will also be used for labels in the rulers if rulers are being displayed.

void GraphDestroy ( ci, hGraph )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

Call this function to destroy the graph.  This is usually done in the XfmDestroy() function.  After the graph is destroyed, the handle is no longer valid, and all memory used by the graph is freed.

HWND DisplayInit ( ci, hWndParent, wTemplateID, wID, lpfnCallback, dwUserData)

COOLINFO *ci			Cool Edit info structure pointer

WORD wTemplateID		ID of bounding rectangle in dialog box

WORD wID			ID for this display object

DWFARPROC lpfnCallback		Callback for getting display graph information

DWORD dwUserData		User data to be passed to callback

This function creates a display object which is encapsulated in a window.  The callback procedure is used to get the y values for a particular x value in the graph.   Attributes of the graph can be changed via the WM_SETDISPLAYDATA message.  The callback procedure should be defined as:

__declspec(dllexport) DWORD FAR PASCAL DisplayCallback(double fXpos, DWORD dwUserData,

WORD wChartNumber, double * pfYpos)

dwUserData should be set to the COOLINFO structure, or the pointer to your transform’s data.  The chart number is the same as given in the DISPLAY_ADDCHART message, and fXpos ranges between the values given in the DISPLAY_LEFTVALUE and DISPLAY_RIGHTVALUE messages.

Once a display window has been initialized and created, messages can be sent to it through the WM_SETDISPLAYDATA message, with graph number in the low word of wParam, and the message id in the high word of wParam.  The actual value to set  the graph attribute to is in the lParam.

HIWORD(wParam)		lParam

DISPLAY_BAKCOLOR	RGB value of background

DISPLAY_GRIDCOLOR	RGB value of grid

DISPLAY_TEXTCONTROL	Dialog box ID of text control to display under the mouse location information

DISPLAY_XLABEL		Label for X axis

DISPLAY_YLABEL		Label for Y axisDISPLAY_SAMPLERATE	Sample rate of audio data (used in some conversions)

DISPLAY_ADDCHART	Add a new chart (colored line) to the display.  See below for lParam values.

DISPLAY_LINECOLOR	RGB value of this line

DISPLAY_TOPVALUE	Topmost Y value to display in chart

DISPLAY_BOTTOMVALUE	Bottom-most Y value in chart

DISPLAY_LEFTVALUE	Leftmost X value in chart

DISPLAY_RIGHTVALUE	Rightmost X value in chart

For example, to create a graph of time in seconds vs decibels, in the range of 0 to 12 seconds, and -40 to 0 dB, the following statements could be used:

double fLeft=0;

double fRight=12;

double fBottom=-40;

double fTop=0;

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(0,DISPLAY_XLABEL),(DWORD)(LPCSTR)"sec");

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(0,DISPLAY_YLABEL),(DWORD)(LPCSTR)"dB");

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(1,DISPLAY_ADDCHART),DISPLAY_MODE_NORMAL);

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(1,DISPLAY_LINECOLOR),(DWORD)RGB(0,255,0));

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(1,DISPLAY_TOPVALUE),(DWORD)&fTop);

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(1,DISPLAY_BOTTOMVALUE),(DWORD)&fBottom);

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(1,DISPLAY_LEFTVALUE),(DWORD)&fLeft);

SendMessage(lpAmp->hWndDisplay,WM_SETDISPLAYDATA,MAKELONG(1,DISPLAY_RIGHTVALUE),(DWORD)&fRight);

The DISPLAY_ADDCHART’s value can be any of combination of the following:

DISPLAY_MODE_DB		Logarithmic scale on the Y axis.

DISPLAY_MODE_LOG		Logarithmic scale on the X axis.

DISPLAY_MODE_NORMAL		Normal linear scale on X axis (do not use with DISPLAY_MODE_LOG)

DISPLAY_MODE_WRAP		If a value is too high or too low, the graph wraps around

Real-Time Preview (CEPro Only)

If the processing in XfmDo can be done faster than real time, then the result may be previewed in real time before pressing OK.  Preview will basically call XfmDo, but instead of the results being sent into a wave file, they are sent to the audio output.  The preview can also adjust in real time to parameter changes in the settings dialog by either automatically cancelling out of the XfmDo procedure (via the ProgressCanceled flag) and restarting with new parameters in the user defined structure, or by calling XfmUpdate, and allowing XfmUpdate to modify the parameters necessary so the main loop in XfmDo does not even have to exit.

Real Time Preview will only work for Cool Edit Pro.  Preview is not currently supported in Cool Edit 2000.

DWORD PreviewStart ( ci, dwStartFlags, wCallbackID )

COOLINFO *ci			Cool Edit info structure pointer

DWORD dwStartFlags		Any of the flags listed below

WORD wCallbackID		WM_COMMAND ID to call back with notification messages

Call PreviewStart to begin previewing, or to query whether or not the version of Cool Edit being used can do previews (Cool Edit ShareWare for example can not do real-time previews).  PreviewStart is generally called in response to pressing the “Preview” button.  Possible flags to pass in are:

PREVIEW_RESTART_ALWAYS	If set, audio is played again from the beginning if any parameters were updated

PREVIEW_NORESTARTONLOOP	If set, XfmDo sees in infinitely long data stream, which is a looped version of the original

PREVIEW_BUFSIZE125		Minimum buffer size to try is 125ms (may cause choppy playback on some systems)

PREVIEW_BUFSIZE250		Minimum buffer size of 250ms

PREVIEW_BUFSIZE500		Minimum buffer of 500ms (This is normal).  Buffer size determines latency.

PREVIEW_BUFSIZE1000

PREVIEW_BUFSIZE1500

PREVIEW_BUFSIZE2000

PREVIEW_QUERY			Just query whether or not Cool Edit can do previews.  If so, PreviewStart returns TRUE

The following notification messages may be issued in the high word of wParam when sending a WM_COMMAND message with the given wCallbackID in the low word of wParam:

PREVN_STARTED	Preview mode was started successfully.

PREVN_STOPOK 	Preview mode ended normally

PREVN_STOPERR	Preview mode ended because of error - XfmDo() returned non-zero and lParam contains the error number.

PREVN_STOPNOW	You should call PreviewStop in response to this notificaiton.

Following is an example of the WM_COMMAND handling for the preview notification callback ID:

case ID_PREVIEWNOTIFY:

switch (HIWORD(wParam))

{	case PREVN_STOPOK:

case PREVN_STOPERR:

bPreviewing=FALSE;

SetDlgItemText(hWndDlg,IDC_PREVIEW,"&Preview");

break;

case PREVN_STARTED:

SetDlgItemText(hWndDlg,IDC_PREVIEW,"&Stop");

bPreviewing=TRUE;

break;

case PREVN_STOPNOW:

PreviewStop(ci);

break;

}

This code snippet changes the text of the dual-purpose “Preview” button, and also sets a global flag that states whether or not we are currently previewing something.  Elsewhere in the code, the bPreviewing flag can be queried, and XfmUpdate called if we are previewing.

DWORD PreviewStop ( ci )

COOLINFO *ci			Cool Edit info structure pointer

Stop previewing.  The XfmDo procedure is exited via setting *ci->lpProgressCanceled to TRUE.

DWORD PreviewUpdate ( ci, dwFlags)

COOLINFO *ci			Cool Edit info structure pointer

DWORD dwFlags			Any of the flags listed below

If audio is currently being played for preview, and a settings dialog parameter changes, PreviewUpdate can be called to update the various parameters without interrupting playback.

If dwFlags is set to PREVIEW_NORESTART then XfmUpdate is called to update parameters, and the *ci->lpProgressCancelled flag is not set.  XfmUpdate should handle all parameter updating necessary so XfmDo does not need to be exited and called again.  If dwFlags is zero, then XfmDo is cancelled, and called again with the updated parameters.

_declspec(dllexport) BOOL FAR PASCAL XfmUpdate ( ciSource, ciDest, dwCursor)

COOLINFO * ciSource		Cool Edit info structure to copy data from

COOLINFO * ciDest		Cool Edit info structure to copy data to (currently in use by XfmDo)

DWORD dwCursor			Current sample offset being played (for updating properly if data varies over time)

XfmUpdate is used to update live parameters while they are still being used in XfmDo.  If done properly, preview audio can very closely follow the user’s motions in moving the controls in the settings dialog.  Sometimes an internal runtime structure is created during XfmDo which contains all the local variables that control the transform’s progress.  This “runtime” structure can be updated to change XfmDo’s internal processing variables if necessary.  See the Distort sample file for somewhat of an example.

FFTs and Convolution

Some routines are used over and over again in digital audio.  One of the most popular is the FFT, as well as the most popular use for an FFT - digital convolution.  Both these functions are provided in double precision floating point accuracy for the XFM module developer.  The FFT will transform time-domain audio data to a frequency-domain set of amplitudes and phases.  Convolution will basically multiply every sample in an impulse by every sample in the user’s data stream, and is a popular method for implementing large FIR filters.  More information on FFTs and Convolution can be found in DSP related literature.

void LFFT ( ci, hpReData, hpImData, lSize, iDirection )

COOLINFO *ci			Cool Edit info structure pointer

double huge * hpReData		Pointer to array of lSize doubles representing the Real components

double huge * hpImData		Pointer to array of lSize doubles representing the Imaginery components

long  iSize			Size of FFT (number of data points)

int iDirection			1 for FFT, -1 for inverse FFT

Perform an FFT (Fast Fourier Transform)  on the data.  The largest sized FFT supported depends on the amount of RAM the user has, which will be evident when the huge arrays are allocated by your DLL.  On the average, FFT sizes up to 262,144 points can be performed successfully on most systems.  The data is zero-based, so hpReData[0] is the first real value part of the first data point for example.  The number of data points should only have the factors 2,3,4, and 5.  So an FFT of size 10,000 is fine, since that 10,000 factors to 5 * 5 * 5 * 5 * 4 * 4.  So, any powers of 2, 3, 4, or 5 are also valid by this rule.

After the FFT is performed, the data can be manipulated, or viewed, or whatever.  If it is manipulated in some way (eg changing the phase or amplituded of various frequency components), then the inverse FFT can be performed by setting iDirection to -1.  You must provide any windowing necessary on this data, as the only "Large" FFT function provided is this one.

It us usually necessary to window data first before performing an FFT on it.  A triangular window, for example, is just multiplying the data going into the FFT, sample by sample, by a triangular shaped set of data - that is, the beginning and ending elements are multiplied by near zero, and slowly each element is multiplied by a slightly larger value until at the center point the element is being multiplied by 1.0, then the multiplier goes linearly back down to zero again by the last sample.

void ConvoluteEx ( ci, wBitsPerSample, wChannels, wBlockAlign, dwLoSample, dwHiSample,

bCombine, wNumImpulses, hImpulse, lImpulseSize, lDelaySamples,

szProgressText, dwUser, lpfnReadData, lpfnWriteData,

fLeftAmp, fRightAmp )

COOLINFO *ci			Cool Edit info structure pointer

WORD wBitsPerSample		Must be either 16 or 32 bits per sample.

WORD wChannels			Number of channels of source data that will be convolved

WORD wBlockAlign		Block alignment of source data (bytes per sample)

DWORD dwLoSample		First sample to convolve

DWORD dwHiSample		Last sample to convolve (Samples offsets from dwLoSample to dwHiSample will be

passed into the lpfnReadData and lpfnWriteData functions)

BOOL bCombine			If source audio is stereo, combine into mono data before performing convolution

WORD wNumImpulses		Number of impulses interleaved in hImpulse

HANDLE hImpulseIn		Handle to globally allocated memory block of lImpulseSize*wNumImpulses doubles

long lImpulseSize			Number of samples represented in the impulse block

long lDelaySamples		Number of samples to skip before returning valid data

LPCSTR szProgressText		If given, a progress meter with this text will show progress of convolution

DWORD dwUser			This value will be passed on into the read and write callbacks

FARPROC lpfnReadData		int ReadData(char huge *data, DWORD dwOffset, int iAmount)

FARPROC lpfnWriteData		int WriteData(char huge *data, DWORD dwOFfset, int iAmount)

double fLeftAmp			Final amplification of left channel (1.0 to remain unchanged)

double fRightAmp			Amplification multiplier for right channel.

One of the most popular uses for FFTs is in convolution.  Since this function is so useful, it has been included here so any Transform function may use it.  Impulses of up to about 100,000 samples may be given.  The impulse data consists of an interleaved (eg Left0,Right0,Left1,Right1,Left2,Right2,...) array of doubles generally with values between -1.0 and 1.0.  The number of impulses being interleaved (valid values are 1,2, or 4) must be given in wNumImpulses.  The following types of convolution may be performed (with the symbol (X) standing for convolution):

Source Channels	Number of Impulses	Combine	Action Performed

1	1	TRUE/FALSE	Out = In (X) Impulse

2	1	FALSE	OutLeft = InLeft (X) Impulse

OutRight = InRight (X) Impulse

2	1	TRUE	OutLeft =((InLeft + InRight) /2) (X) Impulse

OutRight = ((InLeft + InRight) /2) (X) Impulse

2	2	FALSE	OutLeft = InLeft (X) Impulse1

OutRight = InRight (X) Impulse2

2	2	TRUE	OutLeft =((InLeft + InRight) /2) (X) Impulse1

OutRight = ((InLeft + InRight) /2) (X) Impulse2

2	4	FALSE	OutLeft=InLeft (X) Impulse1 + InRight (X) Impulse3

OutRight=InRight (X) Impulse2 + InLeft (X) Impulse4

A custom ReadData and WriteData function should be passed in (see below for a good example).  This function can be a bit tricky to use, but it is extremely powerful.  Convolution entails mulltiplying every sample in the source by every sample in the impulse.  If the impulse is symmetric, then the lDelaySamples should be set to 1/2 the length of the impulse.  For example, a certain filter may have a 200 point impulse symmetric about 100.  In this case, set lDelaySamples to 100, otherwise the audio will be delayed by 100 samples.  If the impulse is the impulse of a reverb or echo, then no delay is necessary, except perhaps the delay to the primary (first) echo.  Please refer to text on digital signal processing for more information about impulses and convolution for more details.

The Read and Write routines take offsets and amounts in bytes, so they must be converted to samples before calling the Read and Write routines of Cool Edit.  The dwUser parameter below is the same as passed into ConvoluteEx, and should be the file ID (ci->dwFileID).  Below are examples of Read and Write routines that can be passed into ConvoluteEx:

__declspec(dllexport) DWORD FAR PASCAL ConvReadData(DWORD dwUser, char * pData,

long lOffset, long lAmount)

{ 	COOLINFO * ci;

WORD wFlags=RWEX_CLIP;

ci=(COOLINFO *)dwUser;

wFlags|=(ci->wChannels==1)?RWEX_MONO:RWEX_STEREO;

wFlags|=(ci->wBitsPerSample==32)?RWEX_32BIT:RWEX_16BIT;

return ReadDataEx(ci,pData,lOffset/ci->wBlockAlign,lAmount/ci->wBlockAlign,wFlags)*ci->wBlockAlign;

}

__declspec(dllexport) DWORD FAR PASCAL ConvWriteData(DWORD dwUser, char * pData,

long lOffset, long lAmount)

{ 	COOLINFO * ci;

WORD wFlags=RWEX_CLIP;

ci=(COOLINFO *)dwUser;

wFlags|=(ci->wChannels==1)?RWEX_MONO:RWEX_STEREO;

wFlags|=(ci->wBitsPerSample==32)?RWEX_32BIT:RWEX_16BIT;

return WriteDataEx(ci,pData,lOffset/ci->wBlockAlign,lAmount/ci->wBlockAlign,wFlags)*ci->wBlockAlign;

}

Miscellaneous Helper Functions

The following functions are some little extras.  The first three are for calling other XFMs from within your XFM module.  The CenterDialog function can always be called from within WM_INITDIALOG…

HANDLE CreateXfmVars ( ci, cszXfmName )

COOLINFO *ci			Cool Edit info structure pointer

LPCSTR cszXfmName		Name of the other effects module

Other effects modules can be used from within your effects module if they exist.  For example, you can possibly use Cool Edit's Filter or Amplify functions from within your own.  But since the internal structures for these functions may change over time, you are better off just using other XFM modules that you have created.  When called, the external structure is initialized to its defaults (from the other module's XfmInit() function).

Returns:  Handle to be used in subsequent calls for identifying the external effect module's data structure

void SetXfmVar ( ci, hVars, iOffset, iLength, lpvMem )

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hVars			Handle for use with external transforms (from CreateXfmVars())

int iOffset			Offset into transform's structure

int iLength			Length of data in transform's structure

void * lpvMem			Memory to copy to transform's structure

Set some data in another effect module's user defined data structure.  After the data is set, the effect may be called using CallXfm().

long CallXfm ( ci, cszXfmName, hVars,

dwLoSamp, dwHiSamp, bShowMeter )

COOLINFO *ci			Cool Edit info structure pointer

LPCSTR cszXfmName		Name of the other effects module

HANDLE hVars			Handle for use with external transforms (from CreateXfmVars())

DWORD dwLoSamp		Low sample value to work with

DWORD dwHiSamp		High sample value to work with

BOOL bShowMeter			Should the called effect show its progress meter?

Actually call the other effect.  The effect will work with samples from dwLoSamp on up to dwHiSamp.  If your function is displaying a progress meter, you will want to set bShowMeter to FALSE so the effect you are calling doesn't try to display its own progress meter as well (which would end in total confusion and chaos).

Returns:  Value depends on function being called.  It is the return value from XfmDo() of the effect being called.

void GetTempName ( ci, szThree, szFilename )

COOLINFO *ci			Cool Edit info structure pointer

LPSTR szThree			Three letter identifier for type of temporary file

LPSTR szFilename			Complete filename of temporary file

This function returns the name of a valid unique temporary file name in szFilename.  The user's choice of temporary directory in Cool Edit is used as the directory for this file.

DWORD GetIniString ( ci, szGroup, szEntry, szDefault, szReturn, dwSize )

COOLINFO *ci			Cool Edit info structure pointer

LPCSTR szGroup			Ini Group (the name you find in brackets)

LPCSTR szEntry			Key name in group

LPCSTR szDefault			Default value to return if key doesn’t exist.

LPSTR szReturn			Returned string

DWORD dwSize			Length of return buffer

This function works just like GetPrivateProfileString, except that the .ini file used is that specified in ci->szIniFile.  You can use Cool Edit’s INI functions on your own .ini file by following the example for this function in xfms99dll.c.  There is no limit to the size of an INI file using these functions (which is why they were written in the first place – to get around the 64K limit of Windows’ INI file functions).

DWORD WriteIniString ( ci, szGroup, szEntry, szString)

DWORD WriteIniStringFile ( ci, szGroup, szEntry, szString, szFile)

COOLINFO *ci			Cool Edit info structure pointer

LPCSTR szGroup			Ini Group (the name you find in brackets)

LPCSTR szEntry			Key name in group

LPSTR szString			String to write under szEntry in section szGroup of the INI file.

This function works just like WritePrivateProfileString, except that the .ini file used is that specified in ci->szIniFile.  You can use Cool Edit’s INI functions on your own .ini file by using WriteIniStringFile instead.   It is OK for files to be larger than 64K if using these functions.

void CenterDialog( ci, hWndDlg )

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

Call this function in the WM_INITDIALOG section of your dialog's settings window to center the dialog with respect to the Cool Edit window that called it.

Definitions (xfmspro.h file)

#define XFM_MONO8	1

#define XFM_STEREO8 	2

#define XFM_MONO16	4

#define XFM_STEREO16	8

#define XFM_MONO32	16

#define XFM_STEREO32	32

//#define XFM_HELPFILE "waycool.hlp"

typedef DWORD (CALLBACK*      DWFARPROC)();

typedef void  (CALLBACK*      VFARPROC)();

typedef struct xfmquery_tag

{	char szName[80];  		// Appears in Menu (without elipsis...)

char szCopyright[80];

char szToolHelp[80];

WORD wSupports;		// OR's of XFM_MONO16, XFM_STEREO16, XFM_MONO8, XFM_STEREO8

DWORD dwFlags;		// Any of the XF_ flags OR'd together

// Must have either XF_TRANSFORM or XF_GENERATE

// May have XF_HASPRESETS, XF_HASSCRIPTS

DWORD dwUserDataLength;		// Length of transform's internal data

char szStructDef[96];	// Array of chars representing types in user data area

char szPresetDef[96];	// Array of chars 'y' to include in presets, 'n' to ignore.

WORD wExtra;			// Number of extra bytes to follow

} XFMQUERY;

#define XFM_VALIDLIBRARY 1160

#define XFM_VALIDLIBRARY98 1157

#define XF_TRANSFORM    		0x0001	// This function transforms highlighted audio

#define XF_GENERATE		0x0002	// This function generates new pcm wave data

#define XF_USESPRESETSAPI	0x0004	// Has a presets box (and uses the presets/scripts API)

#define XF_USESGRAPHAPI		0x0008	// Uses the built in graph control functions

#define XF_MUSTHIGHLIGHT 	0x0010  	// If set, function only enabled if a selection is highlighted (normal)

#define XF_MUSTHAVECLIP    	0x0020  	// If set, function is grayed if nothing on clipboard

#define XF_MODIFIESTOENDOFVIEW  0x0040  	// Set this if function modifies data outside user's given selection

#define XF_MODIFIESTOENDOFFILE  	0x0080 	// Set this if function modifies data outside user's given selection

#define XF_USESFFTAPI		0x0100 	// Uses Cool's FFT functions

#define XF_NOSINGLEEDIT		0x0200 	// Do not include if editing only one channel

#define XF_ANALYZE		0x0400 	// This function analyzes data

#define XF_MODIFIESOUTSIDESEL	0x2000 	// Set this if function modifies data outside selection, and fill dwModifyLeft and

// dwModifyRight upon exiting XfmSetup

#define XF_SYSTEM		0x4000 	// System function - does not show up in drop-down list

#define XF_PREVIEW		0x8000 	// Preview API is being used

#define XF_USES32BITAPI		0x10000	// Uses lpReadEx and lpWriteEx and NOT lpReadData/lpWriteData

#define XF_AUTOCROSSFADE	0x20000 	// Start and End (as defined by dwLoSample/dwHiSample) will be

// crossfaded automatically roughly 200 samples (Uses 32-bit API)

#define XF_MULTIWAVEFORM	0x40000 	// Set if this is an XFM that accepts more than one wave stream

#define XF_AUTOOK		0x80000000L	// If set, wm_initdialog should send "OK" message to close

#define XF_COLORRED		0x40000000L	// If set, WM_CTLCOLOREDIT should color text red

#define XF_STRETCHES		0x20000000L 	// Set if result is of a different size and time markers

// in that area should be proportionally stretched to match

//#define COOLCALLBACK	_far _pascal

//typedef int (COOLCALLBACK* COOLINTPROC)();

// Flags for new Read and Write functions lpReadEx and lpWriteEx

#define RWEX_MONO				0x01

#define RWEX_STEREO				0x02

#define RWEX_8BIT				0x08

#define RWEX_16BIT				0x10

#define RWEX_32BIT				0x20

#define RWEX_TOLEFT				0x100

#define RWEX_TORIGHT				0x200

#define RWEX_FROMRIGHT				0x400

#define RWEX_FROMLEFT				0x800

#define RWEX_CLIP				0x1000

#define RWEX_DITHER				0x2000

#define WAVEINFO_SOURCE	1	// for dwFlags of COOLWAVEINFO

#define WAVEINFO_DEST		2

typedef struct waveinfo_tag		// Information for a particular waveform if processing multiple waves

{	DWORD	dwFlags;

WORD	wChannels;

WORD	wBitsPerSample;

long	lSampleRate;

DWORD	dwLoSample;

DWORD	dwHiSample;

DWORD	dwFileID;

char	szName[128]; // Cool Edit Pro 1.2 and later

} COOLWAVEINFO;

// Old legacy COOLINFO members are italicized

typedef struct coolinfo_tag

{	WORD wChannels;		// Number of channels

WORD wBitsPerSample;	// Bit size, 8 or 16 for now

WORD wBlockAlign;	// Bytes per sample (eg stereo 16-bit = 4)

long lSamprate;		// Sample rate (8000,11025,22050, etc.)

HANDLE hUserData;	// Handle to specialized transform data, depends on DLL

DWORD dwLoSample;       	// Lowest sample to transform

DWORD dwHiSample;	// Highest sample

FARPROC lpTestFunction;

XFMQUERY FAR *cq;	// Pointer to query struct

// General Purpose Tools

FARPROC lpCenterDialog; 	// (HWND hWndDlg, int iUnused)  Center the dialog box

// Reading and Writing audio data

// OLD CODE - use lpReadEx, and lpWriteEx now.

FARPROC lpReadData;	// (char huge *data,long offset,long amount)

FARPROC lpWriteData;	// (char huge *data, DWORD offset,DWORD nbytes)

// Progress Meter

BOOL FAR *lpProgressCanceled;  	// if TRUE, user hit Cancel button, you must stop processing NOW

FARPROC lpProgressMeter; 	// (DWORD dwCurrent, DWORD dwTotal)  percent done is 100*dwCurrent/dwTotal

FARPROC lpProgressCreate; 	// (LPCSTR szText)  szText is message to indicate type of processing

FARPROC lpProgressDestroy; 	// (void)  Call to remove progress meter

// Presets

char far * szIniFile;    	// Ini file

FARPROC lpPresetsInit;	// (HWND hWndDlg,LPSTR szGroupName)

FARPROC lpHandleID_PRESETNAME;

FARPROC lpHandleID_PRESETS;

FARPROC lpHandleID_ADD;

FARPROC lpHandleID_DEL;

// Graph control

FARPROC lpGraphCreate;

FARPROC lpGraphCount;

FARPROC lpGraphGetPoint;

FARPROC lpGraphGetValueAt;

FARPROC lpGraphDraw;

FARPROC lpGraphClear;

FARPROC lpGraphInverse;

FARPROC lpGraphDestroy;

// OLD CODE -- call lpGraphControlHook in main msg proc for each graph

FARPROC lpGraphHandleWM_LBUTTONDOWN;

FARPROC lpGraphHandleWM_LBUTTONUP;

FARPROC lpGraphHandleWM_LBUTTONDBLCLK;

FARPROC lpGraphHandleWM_MOUSEMOVE;

FARPROC lpGraphSetDblClkScales;

FARPROC lpGraphSetDblClkNames;

FARPROC lpGraphSetDialog;

short iScriptFile;

short iScriptDialogStop;

HFONT hFont; 				// smaller font for dialogs

// for XF_GENERATE type, must be filled out during XfmSetup

BOOL	bReplacesHighlightedSelection;	// if TRUE, highlighted selection is deleted before inserting blanks

DWORD	dwInsertBlankSamples;		// the number of samples to generate

BOOL	bHasCoprocessor;			// if TRUE, use fp, otherwise use table lookups and other speedups

// OLD CODE - should do windowing in XFM module itself, and call lpLFFT or lpConvolute

FARPROC lpFFT; 		// perform n-point FFT (radix-2 for now)

FARPROC lpWindowFFT;

FARPROC lpIWindowFFT;

FARPROC lpSetWindowType;  // 0..5: Triangular, von Hann, Hamming, Blackman, Cosine^2, Blackman-Harris

FARPROC lpGetTempName; 	// get temporary file name

FARPROC lpSetFFT16bit;

FARPROC lpSetFFT8bit;

FARPROC lpSetStereoFFT16bit;

FARPROC lpSetStereoFFT8bit;

FARPROC lpSetStereoFFT16bitInterleaved;

FARPROC lpSetStereoFFT8bitInterleaved;

FARPROC lpGetStereoFFT16bitInterleaved;

FARPROC lpGetStereoFFT8bitInterleaved;

FARPROC lpCreateXfmVars;

FARPROC lpSetXfmVar;

DWFARPROC lpCallXfm;

// OLD CODE - use lpDelayWriteEx, etc.

// Delayed writing

VFARPROC lpDelayWrite;

VFARPROC lpDelayWriteInit;

VFARPROC lpDelayWriteDestroy;

HINSTANCE hInst;

FARPROC lpGraphCopy;

FARPROC lpCutData;

FARPROC lpInsertBlankData;

FARPROC lpDeleteXfmVars;

DWORD dwRightSample;    		// Current viewing screen rightmost sample

DWORD dwLeftSample;		// and leftmost sample

FARPROC lpGraphSetPoint;

DWORD dwExtraFlags;

FARPROC lpLFFT;

DWFARPROC lpConvolute;

DWORD dwModifyLeft;

DWORD dwModifyRight;

// Used for Preview mode

HINSTANCE hInstThisLibrary;

DWFARPROC lpPreviewStart;

DWFARPROC lpPreviewStop;

DWFARPROC lpPreviewUpdate;

DWFARPROC lpDisplayInit;

DWORD dwTimeOffset;

DWFARPROC lpLimits;

WORD wInvalidEntry;

HWND hWndDlg;

DWFARPROC lpReadEx;

DWFARPROC lpWriteEx;

WORD wExtraReadFlags;

WORD wExtraWriteFlags;

DWORD dwFileID;

DWFARPROC lpConvoluteEx;

DWFARPROC lpCutSamplesEx;

DWFARPROC lpInsertSamplesEx;

DWFARPROC lpGetMinMax;

DWFARPROC lpDelayWriteInitEx;

DWFARPROC lpDelayWriteEx;

VFARPROC  lpDelayWriteDestroyEx;

char far * szHelpFile;    // Help file for main Cool Edit

FARPROC lpProgressSetText; // (LPCSTR szText) to change progress text at any time

FARPROC lpGraphControlHook;

FARPROC lpDialogHook;

int iNumWaveforms;

COOLWAVEINFO * pWaveforms;

FARPROC lpRegisterPresets;

FARPROC lpRegisterGraph;

FARPROC lpGraphSetProperty;

DWFARPROC lpGetInfo;

DWFARPROC lpGetIniString;

DWFARPROC lpWriteIniString;

} COOLINFO;

typedef struct registergraph_tag

{	DWORD dwSize;

HWND hWndDlg;

UINT uiNotify;

UINT uiRect;

UINT uiDisplayText;

WORD hGraph;

} REGISTERGRAPH;

#define GRAPHN_LBUTTONDOWN	201

#define GRAPHN_RBUTTONDOWN	202

#define GRAPHN_BUTTONUP	203

#define GRAPHN_MOUSEMOVE	204

#define GRAPHN_DBLCLK		205

// Graph Properties

//#define GRAPHP_SPLINES		1	// non-zero=splines, 0=linear

//#define GRAPHP_RULERS		2	// non-zero=draw rulers

//#define GRAPHP_GRID		3	// non-zero=draw grid

// Struct passed to RegisterPresets

typedef struct registerpresets_tag

{	DWORD dwSize;			// size of REGISTERPRESETS struct

char szGroupName[64];

HWND hWndDlg;

UINT uiListBox;

UINT uiAddButton;

UINT uiDelButton;

UINT uiNotify;

UINT uiRect;

char szPresetHeading[32];

} REGISTERPRESETS;

// Sent if using new presets format, RegisterPresets and PresetsInitEx

#define PRESETN_ADDING		101

#define PRESETN_ADDED		102

#define PRESETN_DELETING	103

#define PRESETN_DELETED		104

#define PRESETN_CHOOSING	105

#define PRESETN_CHOSEN		106

#define PRESETN_NOPRESETS	107

#define PRESETN_INITIALIZING	108

#define PRESETN_INITIALIZED	109

#define PRESETREPLY_CANCEL	86

// Used in PreviewStart, just to see if the version of Cool supports Preview or not

#define PREVIEW_QUERY			16384

// Used in PreviewUpdate - if set, XfmDo is not called during update, but XfmUpdate IS called,

// otherwise XfmDo is called every time PreviewUpdate is called

#define PREVIEW_NORESTART 		1

// Used in PreviewStart - if set, PREVIEW_NORESTART is ignored, plus XfmDo is called with data

// starting at the beginning of the highlight instead of data at the current location.  This flag

// makes sense only if PREVIEW_NORESTARTONLOOP is not specified in PreviewStart.

#define PREVIEW_RESTART_ALWAYS 	4

// Used in PreviewStart - if set, XfmDo sees an infinite signal which is the looped version

// of the highlighted selection, otherwise XfmDo is called repeatedly for each loop

#define PREVIEW_NORESTARTONLOOP 	2

// If 0 is passed to all Preview functions for the flags, XfmDo is called whenever Update is called,

// and every time the selection loops.  During update, read pointer is not reset to the start of

// selection, but stays wherever it left off before the call to Update.

#define PREVIEW_BUFSIZE125		0x10

#define PREVIEW_BUFSIZE250		0x00

#define PREVIEW_BUFSIZE500		0x20

#define PREVIEW_BUFSIZE1000		0x30

#define PREVIEW_BUFSIZE1500		0x40

#define PREVIEW_BUFSIZE2000		0x50

// Do not use, it's the mas for the above sizes

#define PREVIEW_BUFSIZEMASK		0x70

// Flags that are used in the HIWORD(wParam) of the callback command message

// To use preview, you must fill the ci->hWndDlg during WM_INITDIALOG

#define PREVN_STARTED 1		// Preview mode was started successfully

#define PREVN_STOPOK 2		// Preview mode ended normally

#define PREVN_STOPERR 3 		// Preview mode ended because of error - XfmDo() returned non-zero

// and lParam contains the error number.

#define PREVN_STOPNOW 4 	// You must call PreviewStop in response to this message!!!

// Display Analysis Control

#define WM_SETDISPLAYDATA WM_USER+100

#define DISPLAY_ADDCHART 	1

#define DISPLAY_BAKCOLOR 	2

#define DISPLAY_LINECOLOR 	3

#define DISPLAY_GRIDCOLOR 	4

#define DISPLAY_TOPVALUE 	5

#define DISPLAY_BOTTOMVALUE 	6

#define DISPLAY_LEFTVALUE	7

#define DISPLAY_RIGHTVALUE	8

#define DISPLAY_TEXTCONTROL 	9

#define DISPLAY_SAMPLERATE	10

#define DISPLAY_XLABEL		11

#define DISPLAY_YLABEL		12

#define DISPLAY_MODE_DB	1

#define DISPLAY_MODE_LOG 2

#define DISPLAY_MODE_WRAP 4

Converting from 16-bit transforms

Following are the major differences between 16-bit and 32-bit effects modules:

The __export declarator should be changed to __declspec(dllexport) in all function definitions.

LibMain and _WEP have been replaced by DllMain

All occurrences of int should be changed to short

Message procedures are now of the form MsgProc (HWND, UINT, WPARAM, LPARAM), and all functions using wParam and lParam should be verified.

EXETYPE WINDOWS can be removed from the DEF file.

The user structure should be bracketed with #pragma pack(1) and #pragma pack().

The valid library return value is 1157 for use with both Cool Edit 98 and Cool Edit Pro, or 1160 for just Cool Edit Pro compatibility only.

Obsolete Functions

All of the following functions have been superseded by updated versions.  Most will still function for backwards compatibility, but the new functions should be used instead as soon as possible.  The only functions that have no new version are the windowing functions for working with FFTs.  If you are working with FFTs, then you no doubt know about windowing functions, and can just window the data from within the XFM module directly.

void CopyToDlgItem( hWndDlg, iControl, ci, hVars, lpVars )	OBSOLETE

void CopyFromDlgItem( hWndDlg, iControl, lpci, phVars, plpVars )

HWND hWndDlg			Settings dialog window handle

int iControl			ID of control to store info into (control should be text type and hidden)

COOLINFO * ci		Cool Edit info structure pointer

COOLINFO ** lpci		Pointer to Cool Edit Info Structure pointer

HANDLE hVars			Handle to internally defined structure

HANDLE * phVars			Pointer to handle to internally defined structure

EFFECTVARS * lpVars		Pointer to internally defined structure

EFFECTVARS ** plpVars	Pointer to pointer to internally defined structure

These functions are obsolete.  See the STATE structure under DIALOGMsgProc.

These obsolete functions basically were used to copy the handles and pointers used most frequently to a dialog control, and copy data back from the control to the appropriate handles and pointers.  During the WM_INITDIALOG, the internal data is locked and the pointer to the data is saved in a control's text field.  Whenever any other messages are processed that need this information, it is copied back from the control to the locally declared pointers and handles.  Following is the most common implementation for these two functions.  The control ID passed into these functions should represent a hidden text field in the dialog box.

void CopyToDlgItem(HWND hWndDlg, int iControl,COOLINFO *ci,HANDLE hMyVars,  COMPRESS *lpMyVars)

{	char m[80];

wsprintf(m,"%ld,%d,%ld",lpAmp,hCompress,ci);

SetDlgItemText(hWndDlg,iControl,m);

}

void CopyFromDlgItem(HWND hWndDlg, int iControl,  COOLINFO **ci, HANDLE *hMyVars, COMPRESS **lpMyVars)

{	char m[80];

char *cursor;

GetDlgItemText(hWndDlg,iControl,m,80);

if (!m[0])

{	*lpMyVars=NULL;

*hMyVars=0;

*ci=NULL;

return;

}

cursor=m;

*lpMyVars=(EFFECTVARS *)longfromstring(&cursor);

*hMyVars=(HANDLE)longfromstring(&cursor);

*ci=(COOLINFO *)longfromstring(&cursor);

}

int ReadData( ci, hpData, lOffset, lSize )	OBSOLETE

COOLINFO * ci		Cool Edit info structure pointer

char huge * hpData		Buffer to read into

long lOffset		Offset into user's wave data

long lSize		Number of bytes to read

This function is obsolete (but will still work).  Please use ReadDataEx instead.  Read lSize bytes into the buffer hpData from the user's waveform data starting at lOffset bytes into user's data.  The entire amount is always read.  Data at offsets below zero, or greater than the length of the file are filled with zeroes for 16-bit data, or 128's for 8-bit data. Remember that the this functions works in units of bytes, not samples, and that the formula for converting samples to bytes is Bytes = Samples * wBitsPerSample * wChannels / 8.

Returns:  0 if all went OK, otherwise an error occurred.

int WriteData( ci, hpData, lOffset, lSize )	OBSOLETE

COOLINFO * ci		Cool Edit info structure pointer

char huge * hpData		Buffer to write out

long lOffset		Offset into user's wave data

long lSize		Number of bytes to write

This function is obsolete (but will still work).  Please use WriteDataEx instead.  Write lSize bytes from the buffer hpData to user's waveform data starting at lOffset bytes into user's data.  The entire amount is always written.  Data at offsets below zero are ignored, and data at offsets greater than the length of the file extend the length of the user's file.  Remember that the this functions works in units of bytes, not samples, and that the formula for converting samples to bytes is Bytes = Samples * wBitsPerSample * wChannels / 8.

Returns:  0 if all went OK, otherwise an error occurred.

BOOL PresetsInit( ci, hWndDlg, cszGroupName )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

LPCSTR cszGroupName		Name of function

This should be called in the WM_INITDIALOG section of your dialog's settings window if presets are used.  cszGroupName identifies a unique name for your function which is used as a key in the INI file for keeping the preset information.  This is usually the same as the name of the function given in the XFMQUERY structure.  The szStructDef array of XFMQUERY contains an array of characters representing each of the data items in the user defined structure.  szPresetDef contains an array of 'y' and 'n' characteres which specify whether or not to include the associated user data item in the preset.  For example, a user defined structure with 4 integers (szStructDef="iiii") of which only the first 3 should be saved when added to the presets would have szPresetDef equal to "yyyn".

The dialog box must have the following IDs for various preset controls:

#define ID_PRESETS	1005	// Single selection sorted list box with Notify flag set

#define ID_PRESETNAME	1006	// Edit control for user to enter new preset name to add or delete

#define ID_ADD		325	// Button with text "Add"

#define ID_DEL		1109	// Button with text "Del"

Returns:  0 if no presets yet exist, or the number of presets if some do exist.  (you can cast the BOOL return type to int).

BOOL HandleID_PRESETS ( ci, hWndDlg, cszGroupName, lParam )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

LPCSTR cszGroupName		Name of function

LPARAM lParam			lParam passed to settings dialog

Call this function in response to the WM_COMMAND message when wParam is ID_PRESET.  The function returns TRUE if the user chose a new preset item (double clicked on a name in the presets box).  The StructToDialog() procedure should be called if this is the case to update all the dialog controls with the new data.

case ID_PRESETS:

if ( HandleID_PRESETS( ci, hWndDlg, TRANSFORMNAME, lParam ) )

{	StructToDialog( ci, lpMyVars, hWndDlg );

}

break;

Returns:  TRUE if user chose a new preset item, FALSE otherwise.

void HandleID_ADD ( ci, hWndDlg, cszGroupName )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

LPCSTR cszGroupName		Name of function

Call this function in response to the WM_COMMAND message when wParam is ID_ADD.  The DialogToStruct() function should be called before calling HandleID_ADD() to fill the COOLINFO structure with the information in the dialog box controls.

case ID_ADD:

DialogToStruct(ci, hWndDlg, lpMyVars);

HandleID_ADD(ci, hWndDlg, TRANSFORMNAME);

break;

void HandleID_DEL ( ci, hWndDlg, cszGroupName )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

LPCSTR cszGroupName		Name of function

Call this function in response to the WM_COMMAND message when wParam is ID_DEL.

case ID_DEL:

HandleID_DEL(ci, hWndDlg, TRANSFORMNAME);

break;

void HandleID_PRESETNAME ( ci, hWndDlg )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HWND hWndDlg			Handle of settings dialog

Call this function in response to the WM_COMMAND message when wParam is ID_PRESETNAME.

case ID_PRESETNAME:

HandleID_PRESETNAME(ci, hWndDlg);

break;

void GraphSetDialog ( ci, hGraph, hWndDlg, uiControl, uiDisplay )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

HWND hWndDlg			Handle of settings dialog

UINT uiControl			ID of rectangle control defining boundaries of graph

UINT uiDisplay			ID of static text control for displaying graph point data

This function is obsolete.  The SetDialog portion gets called automatically during RegisterGraph now.  This should be called in the WM_INITDIALOG section of your dialog's settings window to attach dialog controls to the graph.  The graph must know of a rectangle it can be drawn inside of, and of a text control for displaying graph relevant information, and of the handle to the dialog box itself.  uiControl is the ID of a Frame type picture control in the dialog box, while uiDisplay is the ID of a left aligned text control in the dialog box.  The graph will automatically fit itself inside the bounds of the rectangle.

int GraphHandleWM_LBUTTONDOWN ( ci, hGraph, ptCursor )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

POINT ptCursor			Coordinate of mouse down point (gained from lParam)

Call this function in response to the WM_LBUTTONDOWN message in the settings dialog.  The lParam parameter can be converted to a point since the LOWORD of lParam is the x coordinate, and the HIWORD of lParam is the y coordinate.

case WM_LBUTTONDOWN:

{	POINT here;

int whichpoint;

CopyFromDlgItem( hWndDlg, IDC_STORAGE, &ci, &hMyVars, &lpMyVars );

here.x=LOWORD( lParam );

here.y=HIWORD( lParam );

whichpoint=GraphHandleWM_LBUTTONDOWN( ci, lpMyVars->hGraph, here );

}

Returns:  The index of the point that is under the mouse.  A new point may have been created if the user clicked in an empty area of the graph.   This index will always range from 0 to one minus the number of points in the graph.

int GraphHandleWM_LBUTTONUP ( ci, hGraph, ptCursor )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

POINT ptCursor			Coordinate of mouse up point (gained from lParam)

Call this function in response to the WM_LBUTTONUP message in the settings dialog.  See GraphHandleWM_LBUTTONDOWN for more specifics.

Returns:  The index of the point that is under the mouse.

int GraphHandleWM_LBUTTONDBLCLK ( ci, hGraph, ptCursor )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

POINT ptCursor			Coordinate of mouse double click point (gained from lParam)

Call this function in response to the WM_LBUTTONDBLCLK message in the settings dialog.  See GraphHandleWM_LBUTTONDOWN for more specifics.

Returns:  The index of the point that is under the mouse.

int GraphHandleWM_MOUSEMOVE ( ci, hGraph, ptCursor )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

HANDLE hGraph			Handle to graph given by GraphCreate()

POINT ptCursor			Coordinate of mouse point (gained from lParam)

Call this function in response to the WM_MOUSEMOVE message in the settings dialog.  See GraphHandleWM_LBUTTONDOWN for more specifics.

Returns:  The index of the point that is under the mouse.

void SetWindowType ( ci, wType )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

WORD wType			Type of window

Sets the window type when windowing data for an FFT (when using WindowFFT() and IWindowFFT()).  Window types can be any one of the following:

0	Triangular

1	Von Hann

2	Hamming

3	Blackman

4	Cosine Squared

5	Blackman-Harris

The values go from narrowest band but widest sidelobes and gentle slopes to wider bands with smaller sidelobs and steeper slopes.  In general, Hamming is used most often, but Blackman will give a little steeper slopes.

void WindowFFT ( ci, lpData, iSize, bStereo )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

float * lpData			Pointer to FFT prepared data (prepare with the SetFFT... functions)

int iSize				Size of FFT (number of data points)

BOOL bStereo			TRUE for stereo, FALSE for mono

Once data is converted from 16-bit or 8-bit audio to floating point data, the floating point data can be windowed before performing the FFT.  Windowing the data consists of multiplying each sample by the window values.  Samples near the center are multiplied by 1.0 while samples near the end are multiplied by smaller values down to but not including 0.  Windowing the data will give a more precise measurement of the frequencies present, since the data is assumed to be circular.  Please read about FFTs in other material related to Digital Signal Processing for more information.

void IWindowFFT ( ci, lpData, iSize, bStereo )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

float * lpData			Pointer to FFT prepared data (prepare with the SetFFT... functions)

int iSize				Size of FFT (number of data points)

BOOL bStereo			TRUE for stereo, FALSE for mono

Perform the inverse window on FFT data.  Callint IWindowFFT() right after WindowFFT will result in no change in the original data.  Instead of multiplying the data by the window coefficients, the data is divided by the window coefficients.

void FFT ( ci, lpData, iSize, iDirection )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

float * lpData			Pointer to FFT prepared data (prepare with the SetFFT... functions)

int iSize				Size of FFT (number of data points)

int iDirection			1 for FFT, -1 for inverse FFT

Perform an FFT (Fast Fourier Transform)  on the prepared data.  The floating point values will then contain pairs of  {real,imaginery} data starting at index 1, so lpData[1] contains the real part of the first bin, and lpData[2] contains the imaginery part of the first bin, and so on.  The data going in to the FFT should have been prepared from the original audio data using one of the SetFFT... functions.  The largest size of FFT that should be used with this function is a 4096 point FFT, so iSize should never be more than 4096, and must be a power of 2.

After the FFT is performed, the data can be manipulated, or viewed, or whatever.  If it is manipulated in some way (eg changing the phase or amplituded of various frequency components), then the inverse FFT can be performed by setting iDirection to -1, then an inverse Window performed by calling IWindowFFT(), and finaly calling the GetFFT... function to convert the floating point data back to audio data.  Please read books on Digital Signal Processing for more information on the FFT.

void SetFFT16bit ( ci, lpFloats, lpiAudio, iSize )

void SetFFT8bit ( ci, lpFloats, lpcAudio, iSize )	OBSOLETE

COOLINFO *ci			Cool Edit info structure pointer

float *lpFloats			Destination array of floating point values for use with FFT functions

int * lpiAudio			Source array of 16-bit samples

usigned char * lpcAudio		Source array of 8-bit samples

int iSize				Number of samples to prepare/copy.

Fill an array of complex floats with integer audio data.  There will be twice as many floating point numbers generated as iSize since complex numbers require two floats (one for the real part, and one for the imaginery part).  The floats array must be large enough to hold iSize*2+1  floating point values (the +1 is because the array is 1 based, not zero based).  iSize integer samples from lpiAudio will be converted to iSize complex floating point values in lpFloats.

If preparing 8-bit audio for an FFT, the SetFFT8bit function is used, and character data is read instead of integer data.

void SetStereoFFT16bit ( ci, lpFloats, lpiLeft, lpiRight, iSize )	OBSOLETE

void SetStereoFFT8bit ( ci, lpFloats, lpcLeft, lpcRight, iSize )

COOLINFO *ci			Cool Edit info structure pointer

float *lpFloats			Destination array of floating point values for use with FFT functions

int * lpiLeft			Source array of 16-bit samples for left chananel

int * lpiRight			Source array of 16-bit samples for right channel

usigned char * lpcLeft		Source array of 8-bit samples for left chananel

unsigned char * lpcRight		Source array of 8-bit samples for right channel

int iSize				Number of samples to prepare/copy.

Two channels can be converted to one set of floating point data at a time for more efficiency.  This way, two FFTs can be performed in parallel.

void SetStereoFFT16bitInterleaved ( ci, lpFloats, lpiAudio, iSize )	OBSOLETE

void SetStereoFFT8bitInterleaved ( ci, lpFloats, lpcAudio, iSize )

COOLINFO *ci			Cool Edit info structure pointer

float *lpFloats			Destination array of floating point values for use with FFT functions

int * lpiAudio			Source array of 16-bit samples, left/right interleaved

usigned char * lpcAudio		Source array of 8-bit samples, left/right interleaved

int iSize				Number of samples to prepare/copy.

Two channels can be converted to one set of floating point data at a time for more efficiency.  This way, two FFTs can be performed in parallel.  The source data is interleaved left sample 0, right sample 0, left sample 1, right sample 1, ...

void GetStereoFFT16bitInterleaved ( ci, lpFloats, lpiAudio, iSize, iOp)	OBSOLETE

void GetStereoFFT8bitInterleaved ( ci, lpFloats, lpcAudio, iSize, iOp )

COOLINFO *ci			Cool Edit info structure pointer

float *lpFloats			Source array of floating point values for use with FFT functions

int * lpiAudio			Destination array of 16-bit samples, left/right interleaved

usigned char * lpcAudio		Destination array of 8-bit samples, left/right interleaved

int iSize				Number of samples to prepare/copy.

int iOp				Operation - set this to zero.

Convert floating point FFT data (data gained after doing an inverse FFT) back to interleaved audio.

To convert FFT data back that was set using SetFFT16bit, simply convert every other floating point value starting with offset 1.  So IntegerSample[0] = (int)lpFloats[1], IntegerSample[1] = (int)lpFloats[3], IntegerSample[2] = (int)lpFloats[5] and so on until you have IntegerSamploe[iSize-1] = (int)lpFloats[(iSize-1)*2+1]